# Project Images

# Demonstration Video

Demonstration Video

SmartSort Demonstration Video (CSE461 Project).MOV

# Web Dashboard (http://10.179.161.248/)

# Pin Layout

# Arduino Nano

IR 5-Array Panel Sensor: D8 - D12
Ultrasonic Left Bin: 
	D2: Trig
	D3: Echo
Ultrasonic Right Bin:
	D4: Trig
	D5: Echo
ESP32S3 Communication:
D6 (RX): GPIO 2 (TX)
D7 (TX): GPIO 1 (RX)

# Servo Driver Layout

CH 0: Left Bin Servo

CH 12: Right Bin Servo

CH 10: ARM
CH 11: Elbow

CH 4: Base

CH 1: Gripper

# Robotic ARM Measurements

CH 0:

Max Ticks: 470

Base: x45

Latch Open: x120

CH 12:

Max Ticks: 470

Base: y145

Latch Open: y70

Initial Position:

b70

a170

e45

G70

Picking Position:

e65

a132

g180(gripped)

Bin Right (Green):

a170

e45

b10

g70 (Release)

Bin Left (Red):

a170

e45

b110

g70 (Release)

Back to Initial

# Servo Adjustment Code

#include <Wire.h>

#include <Adafruit_PWMServoDriver.h>

Adafruit_PWMServoDriver pwm = Adafruit_PWMServoDriver(0x40);

// --- Joint Configuration Structure with Independent Tick Bounds ---

struct Joint {

uint8_t channel;    // PCA9685 Channel

char cmdLetter;     // Single-letter command

const char* name;   // Display name

int minTicks;       // Individual 0 deg pulse limit

int maxTicks;       // Individual 180 deg pulse limit

int currentAngle;   // Current angle tracker

int homeAngle;      // Default home position

int stepDelay;      // Speed in ms per degree (Lower = Faster)

bool isRelaxed;     // Power status

};

// 6 Servos with calibrated tick limits and updated initial positions

Joint joints[] = {

// ch, cmd, name,           minT, maxT, initA, homeA, stepDelay, isRelaxed

{   4, 'b', "Base",          150,  550,    70,    70,        15,     false }, // MG996R (CH 4)

{  10, 'a', "Arm",           150,  550,   170,   170,        15,     false }, // MG996R (CH 10)

{  11, 'e', "Elbow",         150,  500,    45,    45,        15,     false }, // SG90/MG996R (CH 11)

{   1, 'g', "Gripper",       150,  500,    70,    70,         8,     false }, // Gripper (CH 1)

{   0, 'x', "SG90 Latch 1",  140,  470,    50,    50,         6,     false }, // SG90 (CH 0: Max 470, Home 50, Open 140)

{  12, 'y', "SG90 Latch 2",  140,  470,   145,   145,         6,     false }  // SG90 (CH 12: Max 470, Home 145, Open 20)

};

const int NUM_JOINTS = sizeof(joints) / sizeof(joints[0]);

// Pick & Place Keypoints

const int PICK_ELBOW     = 65;   // Extended first

const int PICK_ARM       = 132;  // Lowered second

const int GRIP_CLOSE     = 180;

const int GRIP_OPEN      = 70;

const int DROP_BASE_G    = 10;   // Green Bin

const int DROP_BASE_R    = 110;  // Red Bin

// Latch Positions

const int LATCH1_HOME    = 50;

const int LATCH1_OPEN    = 140;

const int LATCH2_HOME    = 145;

const int LATCH2_OPEN    = 20;

void setup() {

Serial.begin(9600);

Wire.begin();

#if defined(WIRE_HAS_TIMEOUT)

Wire.setWireTimeout(3000, true);

#endif

pwm.begin();

pwm.setPWMFreq(50);

delay(100);

// Initialize all servos to home positions

for (int i = 0; i < NUM_JOINTS; i++) {

setServoAngle(joints[i], joints[i].homeAngle);

joints[i].currentAngle = joints[i].homeAngle;

joints[i].isRelaxed = false;

}

printHelpMenu();

}

void loop() {

if (Serial.available() > 0) {

String input = Serial.readStringUntil('\n');

input.trim();

if (input.length() == 0) return;

// 1. Automated Sorting Sequences ('g' for Green b10, 'r' for Red b110)

if (input.equalsIgnoreCase("g")) {

executePickAndPlace("GREEN BIN (Drop at b10)", DROP_BASE_G);

return;

}

if (input.equalsIgnoreCase("r")) {

executePickAndPlace("RED BIN (Drop at b110)", DROP_BASE_R);

return;

}

// 2. Return All to Home ('h')

if (input.equalsIgnoreCase("h") || input.equalsIgnoreCase("home")) {

goHomeAll();

return;

}

// 3. Quick Latch Helper Commands (open/close)

if (input.equalsIgnoreCase("x open")) {

moveJoint(joints[4], LATCH1_OPEN);

return;

}

if (input.equalsIgnoreCase("x close")) {

moveJoint(joints[4], LATCH1_HOME);

return;

}

if (input.equalsIgnoreCase("y open")) {

moveJoint(joints[5], LATCH2_OPEN);

return;

}

if (input.equalsIgnoreCase("y close")) {

moveJoint(joints[5], LATCH2_HOME);

return;

}

// 4. Live Calibration: Set Max Ticks on the fly (e.g. "max 0 470", "max 12 470")

if (input.startsWith("max ")) {

String params = input.substring(4);

params.trim();

int spaceIdx = params.indexOf(' ');

if (spaceIdx > 0) {

int ch = params.substring(0, spaceIdx).toInt();

int newMax = params.substring(spaceIdx + 1).toInt();

for (int i = 0; i < NUM_JOINTS; i++) {

if (joints[i].channel == ch) {

joints[i].maxTicks = newMax;

Serial.print(F("[CALIBRATED] "));

Serial.print(joints[i].name);

Serial.print(F(" (CH "));

Serial.print(ch);

Serial.print(F(") maxTicks set to: "));

Serial.println(newMax);

setServoAngle(joints[i], joints[i].currentAngle);

return;

}

}

}

Serial.println(F("[ERROR] Format: max <channel> <ticks> (e.g. max 0 470)"));

return;

}

// 5. Live Calibration: Direct Raw Ticks Test (e.g. "t 0 350", "t 12 470")

if (input.startsWith("t ") && input.length() > 2) {

String param = input.substring(2);

param.trim();

int spaceIdx = param.indexOf(' ');

if (spaceIdx > 0) {

int ch = param.substring(0, spaceIdx).toInt();

int targetTicks = param.substring(spaceIdx + 1).toInt();

for (int i = 0; i < NUM_JOINTS; i++) {

if (joints[i].channel == ch) {

targetTicks = constrain(targetTicks, 100, 650);

pwm.setPWM(joints[i].channel, 0, targetTicks);

Serial.print(F("[RAW TICKS] CH "));

Serial.print(joints[i].channel);

Serial.print(F(" -> "));

Serial.println(targetTicks);

return;

}

}

}

}

// 6. Explicit Power-On Commands (e.g. "on all", "on 0", "on 12", "on b")

if (input.startsWith("on ")) {

String target = input.substring(3);

target.trim();

if (target.equalsIgnoreCase("all")) {

for (int i = 0; i < NUM_JOINTS; i++) {

setServoAngle(joints[i], joints[i].currentAngle);

joints[i].isRelaxed = false;

}

Serial.println(F(">>> [POWER ON] All servos re-engaged and holding."));

} else {

for (int i = 0; i < NUM_JOINTS; i++) {

if (target.equalsIgnoreCase(String(joints[i].channel)) || target.equalsIgnoreCase(String(joints[i].cmdLetter))) {

setServoAngle(joints[i], joints[i].currentAngle);

joints[i].isRelaxed = false;

Serial.print(F("[POWER ON] "));

Serial.print(joints[i].name);

Serial.println(F(" re-engaged and locked."));

break;

}

}

}

return;

}

// 7. Power Saving: Disable All Servos ("off all" or "d all")

if (input.equalsIgnoreCase("off all") || input.equalsIgnoreCase("d all") || input.equalsIgnoreCase("relax all")) {

for (int i = 0; i < NUM_JOINTS; i++) {

disableJoint(joints[i]);

}

Serial.println(F(">>> [POWER SAVE] All servos relaxed (0mA idle)."));

return;

}

// 8. Power Saving: Disable Individual Servo (e.g. "d0", "d12", "dx", "dy")

if (input.startsWith("d") || input.startsWith("off ") || input.startsWith("relax ")) {

String target = input;

if (target.startsWith("off ")) target = target.substring(4);

else if (target.startsWith("relax ")) target = target.substring(6);

else target = target.substring(1);

target.trim();

bool found = false;

for (int i = 0; i < NUM_JOINTS; i++) {

if (target.equalsIgnoreCase(String(joints[i].channel)) || target.equalsIgnoreCase(String(joints[i].cmdLetter))) {

disableJoint(joints[i]);

found = true;

break;

}

}

if (!found) Serial.println(F("[ERROR] Joint not found. Use: d0, d12, dx, dy, db etc."));

return;

}

// 9. Manual Angle Movement by Channel Number (e.g. "0 50", "0 140", "12 20", "12 145")

if (isDigit(input.charAt(0)) && input.indexOf(' ') > 0) {

int ch = input.substring(0, input.indexOf(' ')).toInt();

int angle = input.substring(input.indexOf(' ') + 1).toInt();

for (int i = 0; i < NUM_JOINTS; i++) {

if (joints[i].channel == ch) {

moveJoint(joints[i], angle);

return;

}

}

Serial.println(F("[ERROR] Invalid channel number."));

return;

}

// 10. Manual Angle Movement by Letter (e.g. "x50", "x140", "y145", "y20", "b90")

char cmd = tolower(input.charAt(0));

int angle = input.substring(1).toInt();

bool found = false;

for (int i = 0; i < NUM_JOINTS; i++) {

if (joints[i].cmdLetter == cmd) {

moveJoint(joints[i], angle);

found = true;

break;

}

}

if (!found) {

Serial.println(F("[ERROR] Unknown command. Type 'help' for command list."));

}

}

}

// ================= AUTOMATED ROUTINES =================

void executePickAndPlace(const char* label, int dropBaseAngle) {

Serial.print(F("\n>>> STARTING AUTOMATED ROUTINE: "));

Serial.println(label);

Joint &base  = joints[0]; // CH 4

Joint &arm   = joints[1]; // CH 10

Joint &elbow = joints[2]; // CH 11

Joint &grip  = joints[3]; // CH 1

// 1. Verify Home Pose

Serial.println(F("1. Verifying Home pose (b70, a170, e45, g70)..."));

moveJoint(arm, arm.homeAngle);

moveJoint(elbow, elbow.homeAngle);

moveJoint(base, base.homeAngle);

moveJoint(grip, GRIP_OPEN);

delay(300);

// 2. Reach down for object: FIRST Elbow (e65), THEN Arm (a132)

Serial.println(F("2. Extending Elbow first (e65)..."));

moveJoint(elbow, PICK_ELBOW);

delay(150);

Serial.println(F("   Lowering Arm second (a132)..."));

moveJoint(arm, PICK_ARM);

delay(300);

// 3. Grip object

Serial.println(F("3. Gripping object (g180)..."));

moveJoint(grip, GRIP_CLOSE);

delay(500);

// 4. Lift up, retract elbow, rotate base to drop zone

Serial.print(F("4. Lifting (a170, e45) & rotating base to "));

Serial.print(dropBaseAngle);

Serial.println(F(" deg..."));

moveJoint(arm, arm.homeAngle);       // Lift arm first

moveJoint(elbow, elbow.homeAngle);   // Retract elbow

moveJoint(base, dropBaseAngle);      // Rotate base

delay(300);

// 5. Release object

Serial.println(F("5. Dropping object (g70)..."));

moveJoint(grip, GRIP_OPEN);

delay(500);

// 6. Return base to Home

Serial.println(F("6. Returning base to Home (b70)..."));

moveJoint(base, base.homeAngle);

delay(300);

Serial.println(F(">>> [COMPLETE] Arm returned to Home pose.\n"));

}

void goHomeAll() {

Serial.println(F(">>> Moving all joints to Home positions..."));

for (int i = 0; i < NUM_JOINTS; i++) {

moveJoint(joints[i], joints[i].homeAngle);

}

}

// ================= HARDWARE CONTROLS =================

void setServoAngle(Joint &j, int angle) {

int ticks = map(angle, 0, 180, j.minTicks, j.maxTicks);

pwm.setPWM(j.channel, 0, ticks);

}

void disableJoint(Joint &j) {

pwm.setPWM(j.channel, 0, 4096);

j.isRelaxed = true;

Serial.print(F("[RELAXED] "));

Serial.print(j.name);

Serial.print(F(" (CH "));

Serial.print(j.channel);

Serial.println(F(") powered OFF."));

}

void moveJoint(Joint &j, int targetAngle) {

targetAngle = constrain(targetAngle, 0, 180);

if (j.isRelaxed) {

setServoAngle(j, j.currentAngle);

j.isRelaxed = false;

delay(20);

}

if (targetAngle == j.currentAngle) return;

int step = (targetAngle > j.currentAngle) ? 1 : -1;

for (int a = j.currentAngle; a != targetAngle; a += step) {

setServoAngle(j, a);

delay(j.stepDelay);

}

setServoAngle(j, targetAngle);

j.currentAngle = targetAngle;

Serial.print(j.name);

Serial.print(F(" -> "));

Serial.print(targetAngle);

Serial.print(F(" deg ("));

Serial.print(map(targetAngle, 0, 180, j.minTicks, j.maxTicks));

Serial.println(F(" ticks)"));

}

void printHelpMenu() {

Serial.println(F("=================================================="));

Serial.println(F(" 6-Servo Controller (Custom Ticks + Latches)      "));

Serial.println(F("=================================================="));

Serial.println(F("AUTOMATED SORTING:"));

Serial.println(F("  'g'                 -> Pick (e65->a132) & Drop GREEN (b10)"));

Serial.println(F("  'r'                 -> Pick (e65->a132) & Drop RED (b110)"));

Serial.println(F("  'h'                 -> Move all joints Home"));

Serial.println(F("--------------------------------------------------"));

Serial.println(F("LATCH SHORTCUTS:"));

Serial.println(F("  'x open' / 'x close' -> CH 0 to 140 deg / 50 deg"));

Serial.println(F("  'y open' / 'y close' -> CH 12 to 20 deg / 145 deg"));

Serial.println(F("--------------------------------------------------"));

Serial.println(F("MANUAL MOVEMENT:"));

Serial.println(F("  b<deg> / 4 <deg>    -> Base (CH 4)"));

Serial.println(F("  a<deg> / 10 <deg>   -> Arm (CH 10)"));

Serial.println(F("  e<deg> / 11 <deg>   -> Elbow (CH 11)"));

Serial.println(F("  g<deg> / 1 <deg>    -> Gripper (CH 1)"));

Serial.println(F("  x<deg> / 0 <deg>    -> Latch 1 (CH 0: Home 50, Open 140)"));

Serial.println(F("  y<deg> / 12 <deg>   -> Latch 2 (CH 12: Home 145, Open 20)"));

Serial.println(F("--------------------------------------------------"));

Serial.println(F("POWER MANAGEMENT:"));

Serial.println(F("  'off all' / 'd all' -> Cut power to all (0mA)"));

Serial.println(F("  'on all'            -> Re-engage all servos"));

Serial.println(F("  'd0', 'd12', 'd4'   -> Disable single servo"));

Serial.println(F("  'on 0', 'on 12'     -> Re-engage single servo"));

Serial.println(F("=================================================="));

}

# Ultrasonic Test Code

// Pin Definitions

const int LEFT_TRIG_PIN  = 2;

const int LEFT_ECHO_PIN  = 3;

const int RIGHT_TRIG_PIN = 4;

const int RIGHT_ECHO_PIN = 5;

// Function to measure distance in centimeters

float readDistanceCM(int trigPin, int echoPin) {

// Clear the trigger pin

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

// Send a 10µs HIGH pulse

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Read echo pulse duration (timeout set to ~30ms / ~5 meters)

long duration = pulseIn(echoPin, HIGH, 30000);

if (duration == 0) {

return -1.0; // Out of range or sensor disconnected

}

// Speed of sound = 343 m/s = 0.0343 cm/µs

return (duration * 0.0343) / 2.0;

}

void setup() {

Serial.begin(9600);

// Initialize Left Sensor pins

pinMode(LEFT_TRIG_PIN, OUTPUT);

pinMode(LEFT_ECHO_PIN, INPUT);

// Initialize Right Sensor pins

pinMode(RIGHT_TRIG_PIN, OUTPUT);

pinMode(RIGHT_ECHO_PIN, INPUT);

Serial.println("--- Dual Ultrasonic Sensor Test Initialized ---");

delay(1000);

}

void loop() {

// Read Left Sensor

float leftDistance = readDistanceCM(LEFT_TRIG_PIN, LEFT_ECHO_PIN);

// 30ms gap to prevent acoustic echo overlap between sensors

delay(30);

// Read Right Sensor

float rightDistance = readDistanceCM(RIGHT_TRIG_PIN, RIGHT_ECHO_PIN);

// Output results to Serial Monitor

Serial.print("Left: ");

if (leftDistance < 0) {

Serial.print("Out of Range");

} else {

Serial.print(leftDistance, 1);

Serial.print(" cm");

}

Serial.print("  |  Right: ");

if (rightDistance < 0) {

Serial.println("Out of Range");

} else {

Serial.print(rightDistance, 1);

Serial.println(" cm");

}

delay(200); // Measurement refresh rate

}

# IR Sensor Test Code

// Pin mapping from left to right: D8, D9, D10, D11, D12

const int irPins[5] = {8, 9, 10, 11, 12};

void setup() {

Serial.begin(9600);

// Set all sensor pins as inputs

for (int i = 0; i < 5; i++) {

pinMode(irPins[i], INPUT);

}

Serial.println("--- 5-Array IR Sensor Test (Active-HIGH: 1 = ON) ---");

Serial.println("Reading order: [ D8 (Left) | D9 | D10 (Center) | D11 | D12 (Right) ]");

Serial.println("-------------------------------------------------------------------");

delay(1000);

}

void loop() {

int sensorState[5];

Serial.print("Raw: [ ");

for (int i = 0; i < 5; i++) {

sensorState[i] = digitalRead(irPins[i]);

Serial.print(sensorState[i]);

if (i < 4) Serial.print("  ");

}

Serial.print(" ]   Visual: ");

// Active-HIGH logic: 1 = Detected, 0 = Clear

for (int i = 0; i < 5; i++) {

if (sensorState[i] == HIGH) {

Serial.print("[■]"); // Sensor ON / Line detected

} else {

Serial.print("[ ]"); // Sensor OFF / Surface

}

}

Serial.println();

delay(100); // 10Hz refresh rate

}

# TX RX Test Code

# ESP32-S3 Code

#include <WiFi.h>

#include "esp_http_server.h"

// ==========================================

// 1. WI-FI CREDENTIALS

// ==========================================

const char *ssid = "Zarif";

const char *password = "password12";

// ==========================================

// 2. UART PINOUT TO ARDUINO NANO (9600 BAUD)

// ==========================================

#define NANO_RX_PIN 1   // ESP32-S3 RX (Connect to Nano Pin D7)

#define NANO_TX_PIN 2   // ESP32-S3 TX (Connect to Nano Pin D6)

HardwareSerial NanoSerial(1);

httpd_handle_t server = NULL;

// Shared diagnostic counters

volatile int pingsReceivedFromNano = 0;

volatile int webCommandsSent = 0;

String lastMessageFromNano = "None";

// ==========================================

// 3. HTML DASHBOARD (PORT 80)

// ==========================================

const char INDEX_HTML[] = R"rawliteral(

<!DOCTYPE html>

<html>

<head>

<meta charset="UTF-8">

<meta name="viewport" content="width=device-width, initial-scale=1">

<title>ESP32-S3 & Nano Link Test</title>

<style>

body { font-family: -apple-system, sans-serif; background: #0f172a; color: #f8fafc; text-align: center; padding: 20px; }

.card { max-width: 440px; margin: 0 auto; background: #1e293b; border-radius: 12px; padding: 20px; box-shadow: 0 4px 15px rgba(0,0,0,0.4); }

.status-box { background: #064e3b; color: #34d399; padding: 12px; border-radius: 8px; font-weight: bold; margin: 15px 0; }

.metric { display: flex; justify-content: space-between; padding: 8px 0; border-bottom: 1px solid #334155; font-size: 15px; }

.btn { width: 100%; padding: 12px; margin-top: 15px; font-size: 16px; font-weight: bold; background: #2563eb; color: white; border: none; border-radius: 6px; cursor: pointer; }

.btn:hover { background: #1d4ed8; }

</style>

</head>

<body>

<div class="card">

<h2>Communication Test</h2>

<div class="status-box">✓ WEB SERVER ONLINE</div>

<div class="metric"><span>Pings from Nano:</span><strong id="ping-count">0</strong></div>

<div class="metric"><span>Last Nano Msg:</span><strong id="last-msg">None</strong></div>

<div class="metric"><span>Commands Sent:</span><strong id="cmd-count">0</strong></div>

<button class="btn" onclick="sendTestToNano()">Send Test Pulse to Nano</button>

</div>

<script>

function sendTestToNano() {

fetch('/send_test')

.then(res => res.text())

.then(() => updateStatus());

}

function updateStatus() {

fetch('/status')

.then(res => res.json())

.then(data => {

document.getElementById('ping-count').innerText = data.nanoPings;

document.getElementById('last-msg').innerText = data.lastMsg;

document.getElementById('cmd-count').innerText = data.webCmds;

});

}

setInterval(updateStatus, 800);

</script>

</body>

</html>

)rawliteral";

// ==========================================

// 4. HTTP SERVER ROUTING

// ==========================================

esp_err_t index_handler(httpd_req_t *req) {

httpd_resp_set_type(req, "text/html");

return httpd_resp_send(req, INDEX_HTML, HTTPD_RESP_USE_STRLEN);

}

esp_err_t status_handler(httpd_req_t *req) {

char json[128];

snprintf(json, sizeof(json), "{\"nanoPings\":%d,\"lastMsg\":\"%s\",\"webCmds\":%d}",

pingsReceivedFromNano, lastMessageFromNano.c_str(), webCommandsSent);

httpd_resp_set_type(req, "application/json");

httpd_resp_set_hdr(req, "Access-Control-Allow-Origin", "*");

return httpd_resp_send(req, json, HTTPD_RESP_USE_STRLEN);

}

esp_err_t send_test_handler(httpd_req_t *req) {

NanoSerial.write('W'); // 'W' = Web command sent to Nano

webCommandsSent++;

Serial.println("[WEB UI] -> Sent test byte 'W' to Arduino Nano!");

return httpd_resp_send(req, "OK", HTTPD_RESP_USE_STRLEN);

}

void startWebServer() {

httpd_config_t config = HTTPD_DEFAULT_CONFIG();

config.server_port = 80;

httpd_uri_t index_uri     = { .uri = "/",          .method = HTTP_GET, .handler = index_handler,     .user_ctx = NULL };

httpd_uri_t status_uri    = { .uri = "/status",     .method = HTTP_GET, .handler = status_handler,    .user_ctx = NULL };

httpd_uri_t send_test_uri = { .uri = "/send_test",  .method = HTTP_GET, .handler = send_test_handler, .user_ctx = NULL };

if (httpd_start(&server, &config) == ESP_OK) {

httpd_register_uri_handler(server, &index_uri);

httpd_register_uri_handler(server, &status_uri);

httpd_register_uri_handler(server, &send_test_uri);

}

}

void setup() {

Serial.begin(115200);

delay(1000); // Rail stabilization delay

NanoSerial.begin(9600, SERIAL_8N1, NANO_RX_PIN, NANO_TX_PIN);

WiFi.mode(WIFI_STA);

WiFi.begin(ssid, password);

Serial.print("Connecting to Wi-Fi");

int attempts = 0;

while (WiFi.status() != WL_CONNECTED && attempts < 40) {

delay(500);

Serial.print(".");

attempts++;

}

WiFi.setSleep(false);

if (WiFi.status() == WL_CONNECTED) {

Serial.println("\n[SUCCESS] Wi-Fi Connected!");

Serial.print("Web Dashboard: http://");

Serial.println(WiFi.localIP());

startWebServer();

} else {

Serial.println("\n[FAILED] Wi-Fi Connection Timeout!");

}

Serial.println("[SYSTEM READY] Listening for Nano UART triggers...");

}

void loop() {

// Listen for data from Arduino Nano

if (NanoSerial.available() > 0) {

char receivedChar = NanoSerial.read();

if (receivedChar == 'T') {

pingsReceivedFromNano++;

lastMessageFromNano = "T (Ping)";

Serial.printf("[NANO RECEIVED] -> Ping #%d ('T')\n", pingsReceivedFromNano);

// Delay 40ms to let Nano SoftwareSerial enter RX listening mode

delay(40);

NanoSerial.write('P'); // Reply with 'P' (Pong)

Serial.println("[ESP32 SENT] -> Replied 'P' (Pong) to Nano");

}

}

delay(5);

}

# Arduino Nano Code

#include <SoftwareSerial.h>

// SoftwareSerial (Nano RX = Pin 6 from ESP TX, Nano TX = Pin 7 to ESP RX)

SoftwareSerial espSerial(6, 7);

unsigned long lastPingTime = 0;

int pingCounter = 0;

void setup() {

Serial.begin(115200);      // USB Serial monitor to PC

espSerial.begin(9600);     // UART link to ESP32-S3

Serial.println("==========================================");

Serial.println(">>> Arduino Nano UART Diagnostic Ready <<<");

Serial.println("==========================================");

}

void loop() {

// 1. Send an automatic Ping ('T') to ESP32-S3 every 2.5 seconds

if (millis() - lastPingTime >= 2500) {

lastPingTime = millis();

pingCounter++;

Serial.print("\n[NANO] Sending Ping #");

Serial.print(pingCounter);

Serial.println(" ('T') to ESP32-S3...");

// Clear residual buffer and ensure listening

while (espSerial.available() > 0) espSerial.read();

espSerial.listen();

// Send single-byte trigger

espSerial.write('T');

// Wait up to 1 second for Pong response from ESP32

unsigned long start = millis();

char response = ' ';

while (millis() - start < 1000) {

if (espSerial.available() > 0) {

response = espSerial.read();

break;

}

}

if (response == 'P') {

Serial.println(">>> [SUCCESS] Received Pong ('P') from ESP32-S3! Link OK.");

} else if (response == 'W') {

Serial.println(">>> [EVENT] Received Web UI button trigger ('W')!");

} else {

Serial.println(">>> [TIMEOUT] No response received from ESP32-S3.");

}

}

// 2. Check if a Web UI button click arrived from ESP32-S3 outside the ping window

if (espSerial.available() > 0) {

char webMsg = espSerial.read();

if (webMsg == 'W') {

Serial.println("\n🔥 >>> [EVENT] Received Web UI button trigger ('W') from Dashboard!");

}

}

delay(20);

}

# ESP32S3 Machine Learning Resources

Model And Training Dataset Link: Edge Impulse

## Dataset:

## Machine Learning Result:

# ESP32S3 Code

#include <Arduino.h>

#include "esp_camera.h"

// 1. Edge Impulse FOMO Inferencing Library

#include <ESP32S3-Sorter_inferencing.h>

// ==========================================

// 2. CAMERA PINOUT (ESP32-S3 CAM)

// ==========================================

#define PWDN_GPIO_NUM     -1

#define RESET_GPIO_NUM    -1

#define XCLK_GPIO_NUM     15

#define SIOD_GPIO_NUM      4

#define SIOC_GPIO_NUM      5

#define Y9_GPIO_NUM       16

#define Y8_GPIO_NUM       17

#define Y7_GPIO_NUM       18

#define Y6_GPIO_NUM       12

#define Y5_GPIO_NUM       10

#define Y4_GPIO_NUM        8

#define Y3_GPIO_NUM        9

#define Y2_GPIO_NUM       11

#define VSYNC_GPIO_NUM     6

#define HREF_GPIO_NUM      7

#define PCLK_GPIO_NUM     13

static camera_fb_t *fb = NULL;

// ==========================================

// 3. ZERO-COPY RGB565 SIGNAL CALLBACK

// ==========================================

static int raw_feature_get_data(size_t offset, size_t length, float *out_ptr) {

size_t pixel_ix = offset;

const uint8_t *buf = fb->buf;

for (size_t i = 0; i < length; i++) {

uint32_t x = (pixel_ix % EI_CLASSIFIER_INPUT_WIDTH);

uint32_t y = (pixel_ix / EI_CLASSIFIER_INPUT_WIDTH);

// Map 96x96 target coordinates to center 240x240 region

uint32_t src_x = 40 + ((x * 240) / EI_CLASSIFIER_INPUT_WIDTH);

uint32_t src_y = (y * 240) / EI_CLASSIFIER_INPUT_HEIGHT;

uint32_t byte_ix = (src_y * 320 + src_x) * 2;

// Unpack RGB565 to standard 24-bit RGB

uint16_t raw_pixel = (buf[byte_ix] << 8) | buf[byte_ix + 1];

uint8_t r = (raw_pixel >> 8) & 0xF8;

uint8_t g = (raw_pixel >> 3) & 0xFC;

uint8_t b = (raw_pixel << 3) & 0xF8;

out_ptr[i] = (float)(((uint32_t)r << 16) | ((uint32_t)g << 8) | b);

pixel_ix++;

}

return 0;

}

// ==========================================

// 4. DEDICATED INFERENCE TASK (32KB STACK)

// ==========================================

void fomoInferenceTask(void *pvParameters) {

signal_t features_signal;

features_signal.total_length = EI_CLASSIFIER_INPUT_WIDTH * EI_CLASSIFIER_INPUT_HEIGHT;

features_signal.get_data = &raw_feature_get_data;

while (true) {

fb = esp_camera_fb_get();

if (fb) {

uint32_t t_start = millis();

ei_impulse_result_t result = { 0 };

EI_IMPULSE_ERROR res = run_classifier(&features_signal, &result, false);

uint32_t infer_ms = millis() - t_start;

esp_camera_fb_return(fb);

fb = NULL;

if (res == EI_IMPULSE_OK) {

bool detected = false;

// Iterate through detected bounding boxes

for (uint32_t i = 0; i < result.bounding_boxes_count; i++) {

ei_impulse_result_bounding_box_t bb = result.bounding_boxes[i];

// 60% confidence filter

if (bb.value >= 0.60f) {

detected = true;

Serial.printf("[%3ums] DETECTED: >>> [ %s ] <<< (%.1f%%) at [x:%u, y:%u, w:%u, h:%u]\n",

infer_ms, bb.label, bb.value * 100.0f, bb.x, bb.y, bb.width, bb.height);

}

}

if (!detected) {

Serial.printf("[%3ums] Tray Status: >>> [ EMPTY ] <<<\n", infer_ms);

}

} else {

Serial.printf("[ERROR] Classifier failed with code: %d\n", res);

}

}

vTaskDelay(pdMS_TO_TICKS(40)); // Yield to prevent watchdog starvation

}

}

// ==========================================

// 5. SETUP

// ==========================================

void setup() {

Serial.begin(115200);

while (!Serial && millis() < 2000);

delay(500);

Serial.println("\n==============================================");

Serial.println("     ESP32-S3 FOMO OBJECT DETECTOR ONLINE     ");

Serial.println("==============================================");

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer   = LEDC_TIMER_0;

config.pin_d0       = Y2_GPIO_NUM;

config.pin_d1       = Y3_GPIO_NUM;

config.pin_d2       = Y4_GPIO_NUM;

config.pin_d3       = Y5_GPIO_NUM;

config.pin_d4       = Y6_GPIO_NUM;

config.pin_d5       = Y7_GPIO_NUM;

config.pin_d6       = Y8_GPIO_NUM;

config.pin_d7       = Y9_GPIO_NUM;

config.pin_xclk     = XCLK_GPIO_NUM;

config.pin_pclk     = PCLK_GPIO_NUM;

config.pin_vsync    = VSYNC_GPIO_NUM;

config.pin_href     = HREF_GPIO_NUM;

config.pin_sccb_sda = SIOD_GPIO_NUM;

config.pin_sccb_scl = SIOC_GPIO_NUM;

config.pin_pwdn     = PWDN_GPIO_NUM;

config.pin_reset    = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_RGB565; // Direct memory buffer

config.frame_size   = FRAMESIZE_QVGA;     // 320x240

config.fb_count     = 2;

config.fb_location  = CAMERA_FB_IN_PSRAM;

config.grab_mode    = CAMERA_GRAB_LATEST;

if (esp_camera_init(&config) != ESP_OK) {

Serial.println("[FATAL] Camera hardware init failed!");

while (1);

}

// Lock sensor white balance and exposure

sensor_t *s = esp_camera_sensor_get();

s->set_brightness(s, 0);

s->set_contrast(s, 1);

s->set_saturation(s, 1);

s->set_whitebal(s, 1);

s->set_awb_gain(s, 1);

s->set_wb_mode(s, 2);

s->set_exposure_ctrl(s, 1);

s->set_aec2(s, 1);

Serial.println("[OK] Camera Initialized.");

Serial.println("[READY] Starting FOMO Task on Core 1...\n");

// Launch isolated FreeRTOS thread with 32KB stack in PSRAM

xTaskCreatePinnedToCore(

fomoInferenceTask,

"FOMO_Task",

32768,

NULL,

1,

NULL,

1

);

}

void loop() {

vTaskDelay(pdMS_TO_TICKS(1000));

}

# Monkey Patching:

1. Replace Old Library in Arduino

- Close Arduino IDE.

- Go to Documents\Arduino\libraries\ in File Explorer and delete the old ESP32S3-Sorter_inferencing folder.

- Open Arduino IDE → Sketch → Include Library → Add .ZIP Library... → select the new .zip file.

- Compile and upload the sketch!

The reason you couldn't find it is because newer Edge Impulse versions dynamically calculate the arena and allocate it via ei_malloc().

The AllocateTensors() failed error happens because ei_malloc() is trying to allocate the 133 KB+ tensor arena inside the ESP32's internal SRAM (which only has ~100 KB contiguous free memory) instead of PSRAM.

Forcing Edge Impulse to allocate all tensor memory directly in PSRAM resolves this issue.

### Step 1: Open the Porting File

- Open Windows File Explorer and navigate to:

- Documents\Arduino\libraries\ESP32S3-Sorter_inferencing\src\edge-impulse-sdk\porting\espressif\

- Open ei_classifier_porting.cpp in Notepad or VS Code.

# ei_classifier_porting.cpp Code

/* The Clear BSD License

*

* Copyright (c) 2025 EdgeImpulse Inc.

* All rights reserved.

*

* Redistribution and use in source and binary forms, with or without

* modification, are permitted (subject to the limitations in the disclaimer

* below) provided that the following conditions are met:

*

*   * Redistributions of source code must retain the above copyright notice,

*     this list of conditions and the following disclaimer.

*

*   * Redistributions in binary form must reproduce the above copyright

*     notice, this list of conditions and the following disclaimer in the

*     documentation and/or other materials provided with the distribution.

*

*   * Neither the name of the copyright holder nor the names of its

*     contributors may be used to endorse or promote products derived from this

*     software without specific prior written permission.

*

* NO EXPRESS OR IMPLIED LICENSES TO ANY PARTY'S PATENT RIGHTS ARE GRANTED BY

* THIS LICENSE. THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND

* CONTRIBUTORS "AS IS" AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT

* LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A

* PARTICULAR PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL THE COPYRIGHT HOLDER OR

* CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL,

* EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO,

* PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR

* BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER

* IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE)

* ARISING IN ANY WAY OUT OF THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF THE

* POSSIBILITY OF SUCH DAMAGE.

*/

#include "../ei_classifier_porting.h"

#if EI_PORTING_ESPRESSIF == 1

#include <stdarg.h>

#include <stdlib.h>

#include <stdio.h>

#include <cstring>

// Include FreeRTOS for delay

#include <freertos/FreeRTOS.h>

#include <freertos/task.h>

// for millis and micros

#include "esp_timer.h"

#include "esp_idf_version.h"

// memory handling

#include "esp_heap_caps.h"

#define EI_WEAK_FN __attribute__((weak))

EI_WEAK_FN EI_IMPULSE_ERROR ei_run_impulse_check_canceled() {

return EI_IMPULSE_OK;

}

EI_WEAK_FN EI_IMPULSE_ERROR ei_sleep(int32_t time_ms) {

vTaskDelay(time_ms / portTICK_RATE_MS);

return EI_IMPULSE_OK;

}

uint64_t ei_read_timer_ms() {

return esp_timer_get_time()/1000;

}

uint64_t ei_read_timer_us() {

return esp_timer_get_time();

}

void ei_putchar(char c)

{

/* Send char to serial output */

putchar(c);

}

/**

*  Printf function uses vsnprintf and output using USB Serial

*/

__attribute__((weak)) void ei_printf(const char *format, ...) {

static char print_buf[1024] = { 0 };

va_list args;

va_start(args, format);

int r = vsnprintf(print_buf, sizeof(print_buf), format, args);

va_end(args);

if (r > 0) {

printf(print_buf);

}

}

__attribute__((weak)) void ei_printf_float(float f) {

ei_printf("%f", f);

}

// 16-byte aligned allocation targeting PSRAM first, falling back to internal RAM

__attribute__((weak)) void *ei_malloc(size_t size) {

void *ptr = NULL;

// 1. Try allocating in PSRAM with 16-byte alignment (Required for ESP32-S3 vector instructions)

ptr = heap_caps_aligned_alloc(16, size, MALLOC_CAP_SPIRAM | MALLOC_CAP_8BIT);

// 2. Fallback to internal RAM if PSRAM allocation fails

if (!ptr) {

ptr = heap_caps_aligned_alloc(16, size, MALLOC_CAP_INTERNAL | MALLOC_CAP_8BIT);

}

// 3. Final fallback

if (!ptr) {

ptr = malloc(size);

}

return ptr;

}

__attribute__((weak)) void *ei_calloc(size_t nitems, size_t size) {

size_t total_size = nitems * size;

void *p = ei_malloc(total_size);

if (p != NULL) {

memset(p, 0, total_size);

}

return p;

}

__attribute__((weak)) void ei_free(void *ptr) {

free(ptr);

}

#if defined(__cplusplus) && EI_C_LINKAGE == 1

extern "C"

#endif

__attribute__((weak)) void DebugLog(const char* s) {

ei_printf("%s", s);

}

#endif // EI_PORTING_ESPRESSIF == 1

In newer versions of the Edge Impulse library, the arena size is located in model_variables.h instead of model_metadata.h.

The TensorFlow Lite Micro runtime needs extra memory overhead for operator scratchpads and 16-byte tensor alignment. Expanding this value in model_variables.h will solve the AllocateTensors() failed error.

### Step-by-Step Fix

1. Open model_variables.h

- Navigate to:

- Documents\Arduino\libraries\ESP32S3-Sorter_inferencing\src\model-parameters\

- Open model_variables.h in Notepad or VS Code.

# model_variables.h code

/* The Clear BSD License

*

* Copyright (c) 2025 EdgeImpulse Inc.

* All rights reserved.

*

* Redistribution and use in source and binary forms, with or without

* modification, are permitted (subject to the limitations in the disclaimer

* below) provided that the following conditions are met:

*

*   * Redistributions of source code must retain the above copyright notice,

*     this list of conditions and the following disclaimer.

*

*   * Redistributions in binary form must reproduce the above copyright

*     notice, this list of conditions and the following disclaimer in the

*     documentation and/or other materials provided with the distribution.

*

*   * Neither the name of the copyright holder nor the names of its

*     contributors may be used to endorse or promote products derived from this

*     software without specific prior written permission.

*

* NO EXPRESS OR IMPLIED LICENSES TO ANY PARTY'S PATENT RIGHTS ARE GRANTED BY

* THIS LICENSE. THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND

* CONTRIBUTORS "AS IS" AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT

* LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A

* PARTICULAR PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL THE COPYRIGHT HOLDER OR

* CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL,

* EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO,

* PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR

* BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER

* IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE)

* ARISING IN ANY WAY OUT OF THE USE OF THIS SOFTWARE, EVEN IF ADVISED OF THE

* POSSIBILITY OF SUCH DAMAGE.

*/

#include "../ei_classifier_porting.h"

#if EI_PORTING_ESPRESSIF == 1

#include <stdarg.h>

#include <stdlib.h>

#include <stdio.h>

#include <cstring>

// Include FreeRTOS for delay

#include <freertos/FreeRTOS.h>

#include <freertos/task.h>

// for millis and micros

#include "esp_timer.h"

#include "esp_idf_version.h"

// memory handling

#include "esp_heap_caps.h"

#define EI_WEAK_FN __attribute__((weak))

EI_WEAK_FN EI_IMPULSE_ERROR ei_run_impulse_check_canceled() {

return EI_IMPULSE_OK;

}

EI_WEAK_FN EI_IMPULSE_ERROR ei_sleep(int32_t time_ms) {

vTaskDelay(time_ms / portTICK_RATE_MS);

return EI_IMPULSE_OK;

}

uint64_t ei_read_timer_ms() {

return esp_timer_get_time()/1000;

}

uint64_t ei_read_timer_us() {

return esp_timer_get_time();

}

void ei_putchar(char c)

{

/* Send char to serial output */

putchar(c);

}

/**

*  Printf function uses vsnprintf and output using USB Serial

*/

__attribute__((weak)) void ei_printf(const char *format, ...) {

static char print_buf[1024] = { 0 };

va_list args;

va_start(args, format);

int r = vsnprintf(print_buf, sizeof(print_buf), format, args);

va_end(args);

if (r > 0) {

printf(print_buf);

}

}

__attribute__((weak)) void ei_printf_float(float f) {

ei_printf("%f", f);

}

// 16-byte aligned allocation targeting PSRAM first, falling back to internal RAM

__attribute__((weak)) void *ei_malloc(size_t size) {

void *ptr = NULL;

// 1. Try allocating in PSRAM with 16-byte alignment (Required for ESP32-S3 vector instructions)

ptr = heap_caps_aligned_alloc(16, size, MALLOC_CAP_SPIRAM | MALLOC_CAP_8BIT);

// 2. Fallback to internal RAM if PSRAM allocation fails

if (!ptr) {

ptr = heap_caps_aligned_alloc(16, size, MALLOC_CAP_INTERNAL | MALLOC_CAP_8BIT);

}

// 3. Final fallback

if (!ptr) {

ptr = malloc(size);

}

return ptr;

}

__attribute__((weak)) void *ei_calloc(size_t nitems, size_t size) {

size_t total_size = nitems * size;

void *p = ei_malloc(total_size);

if (p != NULL) {

memset(p, 0, total_size);

}

return p;

}

__attribute__((weak)) void ei_free(void *ptr) {

free(ptr);

}

#if defined(__cplusplus) && EI_C_LINKAGE == 1

extern "C"

#endif

__attribute__((weak)) void DebugLog(const char* s) {

ei_printf("%s", s);

}

#endif // EI_PORTING_ESPRESSIF == 1

# Full Code

# ESP32S3 Code

#include <Arduino.h>

#include <WiFi.h>

#include "esp_http_server.h"

#include "esp_camera.h"

// 1. Edge Impulse FOMO Model Header

#include <ESP32S3-Sorter_inferencing.h>

// ==========================================

// 2. WI-FI & UART PINOUT

// ==========================================

const char *ssid = "Zarif";

const char *password = "password12";

#define NANO_RX_PIN 1   // ESP32 RX (from Nano Pin D7)

#define NANO_TX_PIN 2   // ESP32 TX (to Nano Pin D6)

HardwareSerial NanoSerial(1);

httpd_handle_t server = NULL;

// ==========================================

// 3. CAMERA PINOUT (ESP32-S3 CAM)

// ==========================================

#define PWDN_GPIO_NUM     -1

#define RESET_GPIO_NUM    -1

#define XCLK_GPIO_NUM     15

#define SIOD_GPIO_NUM      4

#define SIOC_GPIO_NUM      5

#define Y9_GPIO_NUM       16

#define Y8_GPIO_NUM       17

#define Y7_GPIO_NUM       18

#define Y6_GPIO_NUM       12

#define Y5_GPIO_NUM       10

#define Y4_GPIO_NUM        8

#define Y3_GPIO_NUM        9

#define Y2_GPIO_NUM       11

#define VSYNC_GPIO_NUM     6

#define HREF_GPIO_NUM      7

#define PCLK_GPIO_NUM     13

static camera_fb_t *fb = NULL;

// ==========================================

// 4. TELEMETRY & SYSTEM REGISTERS

// ==========================================

enum SystemState {

STATE_IDLE = 0,

STATE_SAMPLING = 1,

STATE_SORTING = 2,

STATE_FAULT = 3,

STATE_FULL_GREEN = 4,

STATE_FULL_RED = 5

};

volatile SystemState currentState = STATE_IDLE;

volatile unsigned long samplingStartTime = 0;

const unsigned long SAMPLING_DURATION_MS = 5000;

volatile int votesRed = 0;

volatile int votesGreen = 0;

volatile int votesEmpty = 0;

volatile int totalInferences = 0;

volatile float remainingSeconds = 0.0;

volatile float distRedCM = 25.0;   // Left Bin (CH 0)

volatile float distGreenCM = 25.0; // Right Bin (CH 12)

String latestVerdict = "STANDBY";

String statusText = "Awaiting object in chute...";

String lastLogMsg = "Telemetry link established.";

// ==========================================

// 5. ZERO-COPY RGB565 SIGNAL CALLBACK

// ==========================================

static int raw_feature_get_data(size_t offset, size_t length, float *out_ptr) {

size_t pixel_ix = offset;

const uint8_t *buf = fb->buf;

for (size_t i = 0; i < length; i++) {

uint32_t x = (pixel_ix % EI_CLASSIFIER_INPUT_WIDTH);

uint32_t y = (pixel_ix / EI_CLASSIFIER_INPUT_WIDTH);

uint32_t src_x = 40 + ((x * 240) / EI_CLASSIFIER_INPUT_WIDTH);

uint32_t src_y = (y * 240) / EI_CLASSIFIER_INPUT_HEIGHT;

uint32_t byte_ix = (src_y * 320 + src_x) * 2;

uint16_t raw_pixel = (buf[byte_ix] << 8) | buf[byte_ix + 1];

uint8_t r = (raw_pixel >> 8) & 0xF8;

uint8_t g = (raw_pixel >> 3) & 0xFC;

uint8_t b = (raw_pixel << 3) & 0xF8;

out_ptr[i] = (float)(((uint32_t)r << 16) | ((uint32_t)g << 8) | b);

pixel_ix++;

}

return 0;

}

void startSamplingProcess() {

votesRed = 0;

votesGreen = 0;

votesEmpty = 0;

totalInferences = 0;

latestVerdict = "SAMPLING...";

statusText = "Analyzing Object in Chute...";

lastLogMsg = "⚡ IR Trigger -> 5s Vision Sampling Started.";

samplingStartTime = millis();

currentState = STATE_SAMPLING;

}

// ==========================================

// 6. FREERTOS INFERENCE & TELEMETRY THREAD

// ==========================================

void fomoInferenceTask(void *pvParameters) {

signal_t features_signal;

features_signal.total_length = EI_CLASSIFIER_INPUT_WIDTH * EI_CLASSIFIER_INPUT_HEIGHT;

features_signal.get_data = &raw_feature_get_data;

while (true) {

while (NanoSerial.available() > 0) {

String line = NanoSerial.readStringUntil('\n');

line.trim();

if (line.startsWith("U:")) {

int commaIdx = line.indexOf(',');

if (commaIdx > 2) {

distRedCM   = line.substring(2, commaIdx).toFloat();

distGreenCM = line.substring(commaIdx + 1).toFloat();

}

}

else if (line == "D") {

startSamplingProcess();

}

else if (line == "A") {

currentState = STATE_FAULT;

remainingSeconds = 0.0;

latestVerdict = "FAULT: REMOVED";

statusText = "Object pulled away mid-scan!";

lastLogMsg = "⚠️ FAULT: Target removed prematurely. Aborted.";

}

else if (line == "F1") {

currentState = STATE_FULL_GREEN;

remainingSeconds = 0.0;

latestVerdict = "GREEN BIN FULL";

statusText = "Right container (CH 12) <= 3.0 cm!";

lastLogMsg = "🛑 ALERT: Green Bin (Right / CH 12) full.";

}

else if (line == "F2") {

currentState = STATE_FULL_RED;

remainingSeconds = 0.0;

latestVerdict = "RED BIN FULL";

statusText = "Left container (CH 0) <= 3.0 cm!";

lastLogMsg = "🛑 ALERT: Red Bin (Left / CH 0) full.";

}

else if (line == "K") {

currentState = STATE_IDLE;

latestVerdict = "STANDBY";

statusText = "Sort complete. Ready for next item.";

lastLogMsg = "✅ Arm returned to Home. Standby.";

}

}

if (currentState == STATE_SAMPLING) {

unsigned long elapsed = millis() - samplingStartTime;

if (elapsed < SAMPLING_DURATION_MS) {

remainingSeconds = (SAMPLING_DURATION_MS - elapsed) / 1000.0;

fb = esp_camera_fb_get();

if (fb) {

ei_impulse_result_t result = { 0 };

EI_IMPULSE_ERROR res = run_classifier(&features_signal, &result, false);

esp_camera_fb_return(fb);

fb = NULL;

if (res == EI_IMPULSE_OK) {

bool foundItem = false;

for (uint32_t i = 0; i < result.bounding_boxes_count; i++) {

ei_impulse_result_bounding_box_t bb = result.bounding_boxes[i];

if (bb.value >= 0.60f) {

foundItem = true;

if (strcmp(bb.label, "red") == 0)   votesRed++;

if (strcmp(bb.label, "green") == 0) votesGreen++;

}

}

if (!foundItem) votesEmpty++;

totalInferences++;

}

}

} else {

remainingSeconds = 0.0;

currentState = STATE_SORTING;

char verdictCode = 'E';

if (votesRed > votesGreen && votesRed > votesEmpty) {

latestVerdict = "RECYCLABLE (RED)";

statusText = "Solid RED Confirmed";

verdictCode = 'R';

} else if (votesGreen > votesRed && votesGreen > votesEmpty) {

latestVerdict = "BIODEGRADABLE (GREEN)";

statusText = "Solid GREEN Confirmed";

verdictCode = 'G';

} else {

latestVerdict = "EMPTY / UNCERTAIN";

statusText = "No dominant class confirmed";

verdictCode = 'E';

}

NanoSerial.println(verdictCode); // Transmit with newline

NanoSerial.flush();

lastLogMsg = "✅ Scan Complete. Sent verdict '" + String(verdictCode) + "' to Nano.";

}

}

vTaskDelay(pdMS_TO_TICKS(20));

}

}

// ==========================================

// 7. RESPONSIVE DASHBOARD

// ==========================================

const char INDEX_HTML[] PROGMEM = R"rawliteral(

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="UTF-8">

<meta name="viewport" content="width=device-width, initial-scale=1, maximum-scale=1, user-scalable=no">

<title>SmartSort Mission Control</title>

<style>

:root {

--bg: #07090e;

--card-bg: rgba(18, 24, 38, 0.7);

--card-border: rgba(255, 255, 255, 0.08);

--cyan: #00f2fe;

--emerald: #10b981;

--rose: #f43f5e;

--amber: #f59e0b;

--text: #f8fafc;

--text-muted: #64748b;

}

* { box-sizing: border-box; margin: 0; padding: 0; -webkit-tap-highlight-color: transparent; }

body {

font-family: -apple-system, BlinkMacSystemFont, "Segoe UI", Roboto, sans-serif;

background: var(--bg);

background-image: radial-gradient(circle at top right, rgba(0, 242, 254, 0.05), transparent 40%),

radial-gradient(circle at bottom left, rgba(244, 63, 94, 0.05), transparent 40%);

color: var(--text);

min-height: 100vh;

padding: 16px;

display: flex;

justify-content: center;

}

.container { width: 100%; max-width: 440px; }

.brand { display: flex; justify-content: space-between; align-items: center; margin-bottom: 16px; padding: 0 4px; }

.brand-title { font-size: 18px; font-weight: 800; letter-spacing: -0.5px; }

.brand-title span { color: var(--cyan); }

.status-pill {

font-size: 11px;

font-weight: 700;

padding: 4px 10px;

border-radius: 20px;

background: rgba(16, 185, 129, 0.15);

color: var(--emerald);

border: 1px solid rgba(16, 185, 129, 0.3);

display: flex;

align-items: center;

gap: 5px;

}

.status-dot { width: 6px; height: 6px; background: var(--emerald); border-radius: 50%; box-shadow: 0 0 8px var(--emerald); }

.card {

background: var(--card-bg);

backdrop-filter: blur(16px);

-webkit-backdrop-filter: blur(16px);

border: 1px solid var(--card-border);

border-radius: 18px;

padding: 16px;

margin-bottom: 14px;

box-shadow: 0 10px 30px rgba(0, 0, 0, 0.4);

}

.hero-verdict {

padding: 22px 16px;

text-align: center;

border-radius: 16px;

transition: all 0.4s cubic-bezier(0.4, 0, 0.2, 1);

}

.hero-title { font-size: 11px; font-weight: 700; text-transform: uppercase; letter-spacing: 1.5px; opacity: 0.7; }

.hero-val { font-size: 24px; font-weight: 900; margin: 6px 0; letter-spacing: -0.5px; }

.hero-sub { font-size: 13px; opacity: 0.85; }

.state-idle { background: rgba(30, 41, 59, 0.5); border: 1px dashed rgba(255, 255, 255, 0.15); color: #94a3b8; }

.state-sampling { background: linear-gradient(135deg, rgba(79, 70, 229, 0.3), rgba(124, 58, 237, 0.3)); border: 1px solid #818cf8; color: #c7d2fe; box-shadow: 0 0 25px rgba(99, 102, 241, 0.3); }

.state-red { background: linear-gradient(135deg, rgba(185, 28, 28, 0.4), rgba(127, 29, 29, 0.4)); border: 1px solid var(--rose); color: #fecdd3; box-shadow: 0 0 25px rgba(244, 63, 94, 0.3); }

.state-green { background: linear-gradient(135deg, rgba(6, 95, 70, 0.4), rgba(4, 120, 87, 0.4)); border: 1px solid var(--emerald); color: #a7f3d0; box-shadow: 0 0 25px rgba(16, 185, 129, 0.3); }

.state-fault { background: linear-gradient(135deg, rgba(180, 83, 9, 0.4), rgba(146, 64, 14, 0.4)); border: 1px solid var(--amber); color: #fde68a; box-shadow: 0 0 25px rgba(245, 158, 11, 0.3); animation: pulse 0.8s infinite alternate; }

.state-full { background: linear-gradient(135deg, rgba(159, 18, 57, 0.5), rgba(136, 19, 55, 0.5)); border: 1px solid var(--rose); color: #ffe4e6; box-shadow: 0 0 30px rgba(244, 63, 94, 0.4); animation: pulse 0.6s infinite alternate; }

@keyframes pulse { from { opacity: 0.75; transform: scale(0.99); } to { opacity: 1; transform: scale(1); } }

.timer-track { width: 100%; height: 6px; background: rgba(255, 255, 255, 0.08); border-radius: 3px; overflow: hidden; margin-top: 12px; }

.timer-bar { width: 0%; height: 100%; background: linear-gradient(90deg, var(--cyan), #818cf8); transition: width 0.1s linear; }

.gauge-row { margin: 12px 0 6px 0; }

.gauge-meta { display: flex; justify-content: space-between; font-size: 13px; font-weight: 600; margin-bottom: 6px; }

.gauge-bg { width: 100%; height: 12px; background: rgba(255, 255, 255, 0.06); border-radius: 6px; overflow: hidden; position: relative; }

.gauge-fill { height: 100%; border-radius: 6px; transition: width 0.3s cubic-bezier(0.4, 0, 0.2, 1); }

.gauge-green { background: linear-gradient(90deg, #059669, var(--emerald)); }

.gauge-red { background: linear-gradient(90deg, #dc2626, var(--rose)); }

.console-header { display: flex; justify-content: space-between; font-size: 11px; font-weight: 700; color: var(--text-muted); text-transform: uppercase; letter-spacing: 1px; margin-bottom: 8px; }

.console-box {

background: #04060a;

border: 1px solid rgba(255, 255, 255, 0.05);

border-radius: 10px;

padding: 10px;

font-family: ui-monospace, "SF Mono", Menlo, Consolas, monospace;

font-size: 11px;

color: var(--cyan);

height: 95px;

overflow-y: auto;

line-height: 1.5;

}

.modal-overlay {

display: none;

position: fixed;

top: 0; left: 0; width: 100%; height: 100%;

background: rgba(4, 6, 10, 0.85);

backdrop-filter: blur(8px);

z-index: 999;

align-items: center;

justify-content: center;

padding: 20px;

}

.modal-card {

background: #151d2c;

border: 2px solid var(--rose);

border-radius: 20px;

padding: 24px 20px;

width: 100%;

max-width: 380px;

text-align: center;

box-shadow: 0 20px 40px rgba(0, 0, 0, 0.6), 0 0 30px rgba(244, 63, 94, 0.3);

animation: modalPop 0.3s cubic-bezier(0.175, 0.885, 0.32, 1.275);

}

@keyframes modalPop { from { transform: scale(0.85); opacity: 0; } to { transform: scale(1); opacity: 1; } }

.modal-card h3 { font-size: 20px; font-weight: 800; color: #fff; margin-bottom: 8px; }

.modal-card p { font-size: 13px; color: #94a3b8; line-height: 1.5; margin-bottom: 20px; }

.btn-stack { display: flex; flex-direction: column; gap: 10px; }

.btn {

width: 100%;

padding: 14px;

border: none;

border-radius: 12px;

font-size: 14px;

font-weight: 700;

cursor: pointer;

display: flex;

align-items: center;

justify-content: center;

gap: 8px;

}

.btn:active { transform: scale(0.98); }

.btn-manual { background: rgba(255, 255, 255, 0.1); color: #fff; border: 1px solid rgba(255, 255, 255, 0.2); }

.btn-auto-dump { background: linear-gradient(135deg, var(--rose), #be123c); color: #fff; box-shadow: 0 4px 15px rgba(244, 63, 94, 0.4); }

</style>

</head>

<body onclick="initAudio()">

<div class="container">

<div class="brand">

<div class="brand-title">SMART<span>SORT</span></div>

<div class="status-pill">

<div class="status-dot"></div>

<span>AUTONOMOUS</span>

</div>

</div>

<div id="verdictCard" class="card hero-verdict state-idle">

<div class="hero-title">NEURAL VISION STATUS</div>

<div id="verdictText" class="hero-val">STANDBY</div>

<div id="subStatus" class="hero-sub">Awaiting object in chute...</div>

<div class="timer-track">

<div id="timerBar" class="timer-bar"></div>

</div>

</div>

<div class="card">

<div style="font-size: 13px; font-weight: 800; letter-spacing: -0.3px; margin-bottom: 4px;">BIN LEVEL TELEMETRY</div>

<div style="font-size: 11px; color: var(--text-muted); margin-bottom: 12px;">Threshold: Full at &lt; 3.0 cm (5s verify)</div>

<!-- Red Bin (Left / CH 0) -->

<div class="gauge-row">

<div class="gauge-meta">

<span style="color: var(--rose);">🔴 Recyclable (Left / CH 0)</span>

<strong id="distRedText">25.0 cm</strong>

</div>

<div class="gauge-bg">

<div id="fillRed" class="gauge-fill gauge-red" style="width: 0%;"></div>

</div>

</div>

<!-- Green Bin (Right / CH 12) -->

<div class="gauge-row">

<div class="gauge-meta">

<span style="color: var(--emerald);">🟢 Biodegradable (Right / CH 12)</span>

<strong id="distGreenText">25.0 cm</strong>

</div>

<div class="gauge-bg">

<div id="fillGreen" class="gauge-fill gauge-green" style="width: 0%;"></div>

</div>

</div>

</div>

<div class="card">

<div class="console-header">

<span>Hardware Event Bus</span>

<span style="color: var(--cyan);">9600 BAUD</span>

</div>

<div class="console-box" id="termLog">

&gt; System initialized. Awaiting sensor events...

</div>

</div>

</div>

<div class="modal-overlay" id="alertModal">

<div class="modal-card">

<div style="font-size: 36px; margin-bottom: 8px;">🛑</div>

<h3 id="modalTitle">BIN FULL ALERT</h3>

<p id="modalDesc">Container capacity threshold exceeded (&lt; 3.0 cm for 5 consecutive seconds). Choose clearance pathway:</p>

<div class="btn-stack">

<button class="btn btn-auto-dump" id="btnAutoDump" onclick="resolveSystemDump()">

<span>⚙️</span> <span id="autoDumpLabel">Let System Handle It (Auto-Dump)</span>

</button>

<button class="btn btn-manual" onclick="resolveManualEmpty()">

<span>✋</span> <span>I Emptied It (Manual Reset)</span>

</button>

</div>

</div>

</div>

<script>

let audioCtx = null;

let activeFullBin = 0;

function initAudio() {

if (!audioCtx) {

audioCtx = new (window.AudioContext || window.webkitAudioContext)();

}

}

function playAlarm(isFault) {

if (!audioCtx) return;

try {

const osc = audioCtx.createOscillator();

const gain = audioCtx.createGain();

osc.type = isFault ? 'sawtooth' : 'square';

osc.frequency.setValueAtTime(isFault ? 330 : 880, audioCtx.currentTime);

if (!isFault) {

osc.frequency.exponentialRampToValueAtTime(660, audioCtx.currentTime + 0.3);

}

gain.gain.setValueAtTime(0.25, audioCtx.currentTime);

gain.gain.exponentialRampToValueAtTime(0.01, audioCtx.currentTime + 0.4);

osc.connect(gain);

gain.connect(audioCtx.destination);

osc.start();

osc.stop(audioCtx.currentTime + 0.4);

} catch(e){}

}

function triggerHaptics(pattern) {

if ("vibrate" in navigator) {

navigator.vibrate(pattern);

}

}

let lastState = 0;

function pollStatus() {

fetch('/status')

.then(res => res.json())

.then(data => {

document.getElementById('distRedText').innerText = data.dL.toFixed(1) + ' cm';

document.getElementById('distGreenText').innerText = data.dR.toFixed(1) + ' cm';

const pctRed   = Math.max(0, Math.min(100, ((25.0 - data.dL) / 22.0) * 100));

const pctGreen = Math.max(0, Math.min(100, ((25.0 - data.dR) / 22.0) * 100));

document.getElementById('fillRed').style.width = pctRed + '%';

document.getElementById('fillGreen').style.width = pctGreen + '%';

document.getElementById('verdictText').innerText = data.vrd;

document.getElementById('subStatus').innerText = data.sub;

const pctTimer = ((5.0 - data.sec) / 5.0) * 100;

document.getElementById('timerBar').style.width = (data.st === 1 ? pctTimer : (data.st === 2 ? 100 : 0)) + '%';

const card = document.getElementById('verdictCard');

card.className = "card hero-verdict " +

(data.st === 0 ? "state-idle" :

(data.st === 1 ? "state-sampling" :

(data.st === 3 ? "state-fault" :

(data.st === 4 || data.st === 5 ? "state-full" :

(data.vrd.includes("RED") ? "state-red" : "state-green")))));

if (data.st !== lastState) {

if (data.st === 3) {

playAlarm(true);

triggerHaptics([150, 80, 150]);

}

else if (data.st === 4) {

activeFullBin = 1;

document.getElementById('modalTitle').innerText = "🟢 GREEN BIN FULL";

document.getElementById('modalDesc').innerText = "Biodegradable container (Right / CH 12) measured <= 3.0 cm for 5.0s. Select clearance method:";

document.getElementById('autoDumpLabel').innerText = "Auto-Dump Green Chute (Latch 2 / CH 12)";

document.getElementById('alertModal').style.display = 'flex';

playAlarm(false);

triggerHaptics([300, 100, 300, 100, 500]);

}

else if (data.st === 5) {

activeFullBin = 2;

document.getElementById('modalTitle').innerText = "🔴 RED BIN FULL";

document.getElementById('modalDesc').innerText = "Recyclable container (Left / CH 0) measured <= 3.0 cm for 5.0s. Select clearance method:";

document.getElementById('autoDumpLabel').innerText = "Auto-Dump Red Chute (Latch 1 / CH 0)";

document.getElementById('alertModal').style.display = 'flex';

playAlarm(false);

triggerHaptics([300, 100, 300, 100, 500]);

}

lastState = data.st;

}

if (data.log && data.log.length > 0) {

const term = document.getElementById('termLog');

term.innerHTML = "&gt; " + data.log + "<br>" + term.innerHTML;

}

})

.catch(err => console.error(err));

}

function resolveManualEmpty() {

document.getElementById('alertModal').style.display = 'none';

fetch('/reset_state');

}

function resolveSystemDump() {

document.getElementById('alertModal').style.display = 'none';

fetch('/dump?bin=' + activeFullBin);

}

setInterval(pollStatus, 250);

</script>

</body>

</html>

)rawliteral";

// ==========================================

// 8. HTTP ROUTING HANDLERS

// ==========================================

esp_err_t index_handler(httpd_req_t *req) {

httpd_resp_set_type(req, "text/html");

return httpd_resp_send(req, INDEX_HTML, HTTPD_RESP_USE_STRLEN);

}

esp_err_t status_handler(httpd_req_t *req) {

char json[340];

snprintf(json, sizeof(json),

"{\"st\":%d,\"vrd\":\"%s\",\"sub\":\"%s\",\"sec\":%.2f,\"dL\":%.1f,\"dR\":%.1f,\"log\":\"%s\"}",

(int)currentState, latestVerdict.c_str(), statusText.c_str(), remainingSeconds, distRedCM, distGreenCM, lastLogMsg.c_str());

lastLogMsg = "";

httpd_resp_set_type(req, "application/json");

httpd_resp_set_hdr(req, "Access-Control-Allow-Origin", "*");

return httpd_resp_send(req, json, HTTPD_RESP_USE_STRLEN);

}

esp_err_t dump_handler(httpd_req_t *req) {

char param[16];

if (httpd_req_get_url_query_str(req, param, sizeof(param)) == ESP_OK) {

char val[4];

if (httpd_query_key_value(param, "bin", val, sizeof(val)) == ESP_OK) {

if (val[0] == '1') {

NanoSerial.println("1");

lastLogMsg = "⚙️ Auto-Dumping Green Bin (CH 12)...";

} else if (val[0] == '2') {

NanoSerial.println("2");

lastLogMsg = "⚙️ Auto-Dumping Red Bin (CH 0)...";

}

NanoSerial.flush();

}

}

currentState = STATE_IDLE;

latestVerdict = "STANDBY";

statusText = "Dump routine executed. Ready.";

return httpd_resp_send(req, "OK", HTTPD_RESP_USE_STRLEN);

}

esp_err_t reset_state_handler(httpd_req_t *req) {

NanoSerial.println("C");

NanoSerial.flush();

currentState = STATE_IDLE;

latestVerdict = "STANDBY";

statusText = "System manually reset.";

lastLogMsg = "✋ Manual clearance confirmed by user.";

return httpd_resp_send(req, "OK", HTTPD_RESP_USE_STRLEN);

}

void startWebServer() {

httpd_config_t config = HTTPD_DEFAULT_CONFIG();

config.server_port = 80;

httpd_uri_t index_uri   = { .uri = "/",           .method = HTTP_GET, .handler = index_handler,       .user_ctx = NULL };

httpd_uri_t status_uri  = { .uri = "/status",      .method = HTTP_GET, .handler = status_handler,      .user_ctx = NULL };

httpd_uri_t dump_uri    = { .uri = "/dump",        .method = HTTP_GET, .handler = dump_handler,        .user_ctx = NULL };

httpd_uri_t reset_uri   = { .uri = "/reset_state", .method = HTTP_GET, .handler = reset_state_handler, .user_ctx = NULL };

if (httpd_start(&server, &config) == ESP_OK) {

httpd_register_uri_handler(server, &index_uri);

httpd_register_uri_handler(server, &status_uri);

httpd_register_uri_handler(server, &dump_uri);

httpd_register_uri_handler(server, &reset_uri);

}

}

// ==========================================

// 9. SETUP

// ==========================================

void setup() {

Serial.begin(115200);

while (!Serial && millis() < 2000);

delay(500);

NanoSerial.begin(9600, SERIAL_8N1, NANO_RX_PIN, NANO_TX_PIN);

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer   = LEDC_TIMER_0;

config.pin_d0       = Y2_GPIO_NUM;

config.pin_d1       = Y3_GPIO_NUM;

config.pin_d2       = Y4_GPIO_NUM;

config.pin_d3       = Y5_GPIO_NUM;

config.pin_d4       = Y6_GPIO_NUM;

config.pin_d5       = Y7_GPIO_NUM;

config.pin_d6       = Y8_GPIO_NUM;

config.pin_d7       = Y9_GPIO_NUM;

config.pin_xclk     = XCLK_GPIO_NUM;

config.pin_pclk     = PCLK_GPIO_NUM;

config.pin_vsync    = VSYNC_GPIO_NUM;

config.pin_href     = HREF_GPIO_NUM;

config.pin_sccb_sda = SIOD_GPIO_NUM;

config.pin_sccb_scl = SIOC_GPIO_NUM;

config.pin_pwdn     = PWDN_GPIO_NUM;

config.pin_reset    = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_RGB565;

config.frame_size   = FRAMESIZE_QVGA;

config.fb_count     = 2;

config.fb_location  = CAMERA_FB_IN_PSRAM;

config.grab_mode    = CAMERA_GRAB_LATEST;

if (esp_camera_init(&config) != ESP_OK) {

Serial.println("[FATAL] Camera init failed!");

while (1);

}

sensor_t *s = esp_camera_sensor_get();

s->set_brightness(s, 0);

s->set_contrast(s, 1);

s->set_saturation(s, 1);

s->set_whitebal(s, 1);

s->set_awb_gain(s, 1);

s->set_wb_mode(s, 2);

s->set_exposure_ctrl(s, 1);

s->set_aec2(s, 1);

WiFi.mode(WIFI_STA);

WiFi.begin(ssid, password);

int attempts = 0;

while (WiFi.status() != WL_CONNECTED && attempts < 40) {

delay(500);

attempts++;

}

WiFi.setSleep(false);

if (WiFi.status() == WL_CONNECTED) {

Serial.print("📱 Open on phone: http://");

Serial.println(WiFi.localIP());

startWebServer();

}

xTaskCreatePinnedToCore(

fomoInferenceTask,

"FOMO_Task",

32768,

NULL,

1,

NULL,

1

);

}

void loop() {

vTaskDelay(pdMS_TO_TICKS(1000));

}

# Arduino Code

#include <Wire.h>

#include <Adafruit_PWMServoDriver.h>

#include <SoftwareSerial.h>

// ==========================================

// 1. PIN CONFIGURATION & DRIVERS

// ==========================================

// SoftwareSerial: RX = Pin 6 (from ESP32 TX), TX = Pin 7 (to ESP32 RX)

SoftwareSerial espSerial(6, 7);

Adafruit_PWMServoDriver pwm = Adafruit_PWMServoDriver(0x40);

// 5-Array IR Sensor (Active-HIGH: 1 = Detected)

const int irPins[5] = {8, 9, 10, 11, 12};

// Dual Ultrasonic Sensors

const int LEFT_TRIG_PIN  = 2;  // Left Bin: RED / Recyclable

const int LEFT_ECHO_PIN  = 3;

const int RIGHT_TRIG_PIN = 4;  // Right Bin: GREEN / Biodegradable

const int RIGHT_ECHO_PIN = 5;

// Threshold for full bin (including blind spot: 0.0cm to 3.0cm)

const float BIN_FULL_THRESHOLD_CM = 3.0;

const unsigned long FULL_VERIFY_DURATION_MS = 5000; // 5-second sampling verification

// ==========================================

// 2. SERVO & JOINT CONFIGURATION

// ==========================================

struct Joint {

uint8_t channel;

char cmdLetter;

const char* name;

int minTicks;

int maxTicks;

int currentAngle;

int homeAngle;

int stepDelay;

bool isRelaxed;

};

Joint joints[] = {

// ch, cmd, name,                 minT, maxT, initA, homeA, stepDelay, isRelaxed

{   4, 'b', "Base",                150,  550,    70,    70,        15,     false }, // MG996R (CH 4)

{  10, 'a', "Arm",                 150,  550,   170,   170,        15,     false }, // MG996R (CH 10)

{  11, 'e', "Elbow",               150,  500,    45,    45,        15,     false }, // SG90/MG996R (CH 11)

{   1, 'g', "Gripper",             150,  500,    70,    70,         8,     false }, // Gripper (CH 1)

{   0, 'x', "SG90 Red Latch (L)",   140,  470,    50,    50,         6,     false }, // CH 0: Left / Red (Home 50, Open 140)

{  12, 'y', "SG90 Green Latch (R)", 140,  470,   145,   145,         6,     false }  // CH 12: Right / Green (Home 145, Open 20)

};

const int NUM_JOINTS = sizeof(joints) / sizeof(joints[0]);

const int PICK_ELBOW   = 65;

const int PICK_ARM     = 132;

const int GRIP_CLOSE   = 180;

const int GRIP_OPEN    = 70;

const int DROP_BASE_G  = 10;   // Green / Right Bin Base Angle

const int DROP_BASE_R  = 110;  // Red / Left Bin Base Angle

const int LATCH_RED_HOME   = 50;   // CH 0

const int LATCH_RED_OPEN   = 140;

const int LATCH_GREEN_HOME = 145;  // CH 12

const int LATCH_GREEN_OPEN = 20;

// ==========================================

// 3. STATE MACHINE & TIMING

// ==========================================

enum SorterState {

STATE_IDLE,

STATE_WAITING_VERDICT,

STATE_COOLDOWN

};

SorterState currentState = STATE_IDLE;

unsigned long triggerStartTime = 0;

unsigned long lastTelemetryTime = 0;

const unsigned long TIMEOUT_MS = 10000;

unsigned long redFullStartTime = 0;

bool redFullTimerActive = false;

unsigned long greenFullStartTime = 0;

bool greenFullTimerActive = false;

// Forward declarations

void setServoAngle(Joint &j, int angle);

void moveJoint(Joint &j, int targetAngle);

void goHomeAll();

bool isObjectPresent();

float readDistanceCM(int trigPin, int echoPin);

bool verifyBinFullSampling(const char* binName, int trigPin, int echoPin);

bool executePickAndPlace(const char* label, int dropBaseAngle);

void dumpBinLatch(int binIndex);

void setup() {

Serial.begin(115200);

espSerial.begin(9600);

Wire.begin();

#if defined(WIRE_HAS_TIMEOUT)

Wire.setWireTimeout(3000, true);

#endif

for (int i = 0; i < 5; i++) {

pinMode(irPins[i], INPUT);

}

pinMode(LEFT_TRIG_PIN, OUTPUT);

pinMode(LEFT_ECHO_PIN, INPUT);

pinMode(RIGHT_TRIG_PIN, OUTPUT);

pinMode(RIGHT_ECHO_PIN, INPUT);

pwm.begin();

pwm.setPWMFreq(50);

delay(100);

for (int i = 0; i < NUM_JOINTS; i++) {

setServoAngle(joints[i], joints[i].homeAngle);

joints[i].currentAngle = joints[i].homeAngle;

joints[i].isRelaxed = false;

}

Serial.println(F("\n======================================================="));

Serial.println(F(">>> ARDUINO NANO: AUTONOMOUS SORTING ENGINE READY   <<<"));

Serial.println(F("  Left Bin  (CH 0  / D2,D3) = RED / RECYCLABLE         "));

Serial.println(F("  Right Bin (CH 12 / D4,D5) = GREEN / BIODEGRADABLE    "));

Serial.println(F("=======================================================\n"));

}

void loop() {

// 1. UNIFIED UART PACKET DISPATCHER (Fixes the Verdict Drop Bug)

if (espSerial.available() > 0) {

String msg = espSerial.readStringUntil('\n');

msg.trim();

if (msg.length() > 0) {

if (msg == "1") {

dumpBinLatch(1); // Auto-Dump Green Bin (CH 12)

currentState = STATE_IDLE;

return;

}

else if (msg == "2") {

dumpBinLatch(2); // Auto-Dump Red Bin (CH 0)

currentState = STATE_IDLE;

return;

}

else if (msg == "C") {

Serial.println(F("✋ [MANUAL RESET] System cleared and re-armed by user."));

currentState = STATE_IDLE;

return;

}

// Process Classifier Verdicts

else if (currentState == STATE_WAITING_VERDICT) {

char verdict = msg.charAt(0);

Serial.print(F("\n>>> [VERDICT RECEIVED]: "));

Serial.println(verdict);

if (verdict == 'G') {

Serial.println(F("🟢 Classified: BIODEGRADABLE (GREEN / RIGHT BIN)"));

Serial.println(F("🔍 [CAPACITY CHECK] Verifying Right Bin (D4, D5)..."));

if (verifyBinFullSampling("GREEN / RIGHT BIN", RIGHT_TRIG_PIN, RIGHT_ECHO_PIN)) {

Serial.println(F("\n🛑 [BIN FULL CONFIRMED] Green Bin <= 3.0cm!"));

Serial.println(F("🛑 [LOCKED] Arm motion blocked. Alert transmitted to phone."));

espSerial.println("F1");

currentState = STATE_IDLE;

} else {

Serial.println(F("✅ [CAPACITY OK] Green Bin clear. Actuating robotic arm..."));

if (executePickAndPlace("GREEN BIN (b10)", DROP_BASE_G)) {

espSerial.println("K"); // Signal ESP32 sort is complete

currentState = STATE_COOLDOWN;

} else {

currentState = STATE_IDLE;

}

}

}

else if (verdict == 'R') {

Serial.println(F("🔴 Classified: RECYCLABLE (RED / LEFT BIN)"));

Serial.println(F("🔍 [CAPACITY CHECK] Verifying Left Bin (D2, D3)..."));

if (verifyBinFullSampling("RED / LEFT BIN", LEFT_TRIG_PIN, LEFT_ECHO_PIN)) {

Serial.println(F("\n🛑 [BIN FULL CONFIRMED] Red Bin <= 3.0cm!"));

Serial.println(F("🛑 [LOCKED] Arm motion blocked. Alert transmitted to phone."));

espSerial.println("F2");

currentState = STATE_IDLE;

} else {

Serial.println(F("✅ [CAPACITY OK] Red Bin clear. Actuating robotic arm..."));

if (executePickAndPlace("RED BIN (b110)", DROP_BASE_R)) {

espSerial.println("K"); // Signal ESP32 sort is complete

currentState = STATE_COOLDOWN;

} else {

currentState = STATE_IDLE;

}

}

}

else if (verdict == 'E') {

Serial.println(F("⚪ [INFO] Classifier returned EMPTY/UNCERTAIN. Holding Home pose."));

espSerial.println("K");

currentState = STATE_IDLE;

}

}

}

}

// 2. Continuous Ultrasonic Telemetry & Proactive Standby Monitoring (Every 350ms)

if (currentState == STATE_IDLE && millis() - lastTelemetryTime >= 350) {

lastTelemetryTime = millis();

float dL = readDistanceCM(LEFT_TRIG_PIN, LEFT_ECHO_PIN);   // Red

delay(30);

float dR = readDistanceCM(RIGHT_TRIG_PIN, RIGHT_ECHO_PIN); // Green

// Format: "U:<RedCM>,<GreenCM>\n"

espSerial.print("U:");

espSerial.print(dL, 1);

espSerial.print(",");

espSerial.println(dR, 1);

Serial.print(F("[LIVE TELEMETRY] Left (RED): "));

if (dL < 0 || dL > 400) Serial.print(F("Out of Range"));

else { Serial.print(dL, 1); Serial.print(F(" cm")); }

Serial.print(F(" | Right (GREEN): "));

if (dR < 0 || dR > 400) Serial.print(F("Out of Range"));

else { Serial.print(dR, 1); Serial.print(F(" cm")); }

// Proactive Standby Full-Bin Verification (Left / Red)

if (dL >= 0.0 && dL <= BIN_FULL_THRESHOLD_CM) {

if (!redFullTimerActive) {

redFullTimerActive = true;

redFullStartTime = millis();

} else if (millis() - redFullStartTime >= FULL_VERIFY_DURATION_MS) {

Serial.println(F("\n🛑 [PROACTIVE ALERT] Red Bin is FULL (<3cm for 5s)!"));

espSerial.println("F2");

redFullTimerActive = false;

}

Serial.print(F(" [RED FULL SAMPLING]"));

} else {

redFullTimerActive = false;

}

// Proactive Standby Full-Bin Verification (Right / Green)

if (dR >= 0.0 && dR <= BIN_FULL_THRESHOLD_CM) {

if (!greenFullTimerActive) {

greenFullTimerActive = true;

greenFullStartTime = millis();

} else if (millis() - greenFullStartTime >= FULL_VERIFY_DURATION_MS) {

Serial.println(F("\n🛑 [PROACTIVE ALERT] Green Bin is FULL (<3cm for 5s)!"));

espSerial.println("F1");

greenFullTimerActive = false;

}

Serial.print(F(" [GREEN FULL SAMPLING]"));

} else {

greenFullTimerActive = false;

}

Serial.println();

}

// 3. Autonomous State Machine

switch (currentState) {

case STATE_IDLE:

if (isObjectPresent()) {

delay(60); // Debounce

if (isObjectPresent()) {

Serial.println(F("\n-------------------------------------------------------"));

Serial.println(F("⚡ [IR SENSOR DETECT] Object placed in chute!"));

Serial.println(F("📡 [UART] Starting 5s Neural Vision Sampling on ESP32-S3..."));

Serial.println(F("-------------------------------------------------------"));

while (espSerial.available() > 0) espSerial.read(); // Clear buffer

espSerial.println("D"); // Send trigger with newline

espSerial.listen();

triggerStartTime = millis();

currentState = STATE_WAITING_VERDICT;

}

}

break;

case STATE_WAITING_VERDICT:

// Fault Recovery: Object removed mid-scan

if (!isObjectPresent()) {

delay(80);

if (!isObjectPresent()) {

Serial.println(F("\n⚠️ [FAULT DETECTED] Object removed mid-scan! Aborting cycle."));

espSerial.println("A"); // Send abort signal with newline

currentState = STATE_IDLE;

break;

}

}

if (millis() - triggerStartTime > TIMEOUT_MS) {

Serial.println(F("⏱️ [TIMEOUT] Scan timed out. Resetting to Standby."));

currentState = STATE_IDLE;

}

break;

case STATE_COOLDOWN:

// Cooldown: wait until previous item has cleared

if (!isObjectPresent()) {

delay(200);

if (!isObjectPresent()) {

Serial.println(F("[READY] Chute clear. System returned to Standby.\n"));

currentState = STATE_IDLE;

}

}

break;

}

delay(20);

}

// ==========================================

// 4. SENSING & 5-SECOND VERIFICATION

// ==========================================

bool isObjectPresent() {

for (int i = 0; i < 5; i++) {

if (digitalRead(irPins[i]) == HIGH) return true;

}

return false;

}

float readDistanceCM(int trigPin, int echoPin) {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH, 26000);

if (duration == 0) return 99.0;

return (duration * 0.0343) / 2.0;

}

bool verifyBinFullSampling(const char* binName, int trigPin, int echoPin) {

unsigned long start = millis();

int fullSamples = 0;

int totalSamples = 0;

while (millis() - start < FULL_VERIFY_DURATION_MS) {

float dist = readDistanceCM(trigPin, echoPin);

if (dist >= 0.0 && dist <= BIN_FULL_THRESHOLD_CM) {

fullSamples++;

}

totalSamples++;

float elapsedSec = (millis() - start) / 1000.0;

Serial.print(F("  [SAMPLING "));

Serial.print(binName);

Serial.print(F("] "));

Serial.print(elapsedSec, 1);

Serial.print(F("s / 5.0s | Measured: "));

if (dist > 400) Serial.print(F("No Echo"));

else { Serial.print(dist, 1); Serial.print(F(" cm")); }

Serial.print(F(" | Full: "));

Serial.print(fullSamples);

Serial.print(F("/"));

Serial.println(totalSamples);

delay(200);

}

float ratio = (float)fullSamples / (float)totalSamples;

return (ratio >= 0.75);

}

// ==========================================

// 5. ACTUATION & DUMP ROUTINES

// ==========================================

bool executePickAndPlace(const char* label, int dropBaseAngle) {

if (!isObjectPresent()) {

Serial.println(F("⚠️ [ABORT] Object was removed right before pick descent!"));

return false;

}

Serial.print(F("\n>>> STARTING ROBOTIC MOTION: "));

Serial.println(label);

Joint &base  = joints[0]; // CH 4

Joint &arm   = joints[1]; // CH 10

Joint &elbow = joints[2]; // CH 11

Joint &grip  = joints[3]; // CH 1

// 1. Verify Home & Open Gripper

moveJoint(arm, arm.homeAngle);

moveJoint(elbow, elbow.homeAngle);

moveJoint(base, base.homeAngle);

moveJoint(grip, GRIP_OPEN);

delay(150);

// 2. Extend Elbow then Lower Arm

Serial.println(F("  -> 1. Extending Elbow (e65)..."));

moveJoint(elbow, PICK_ELBOW);

delay(100);

Serial.println(F("  -> 2. Lowering Arm (a132)..."));

moveJoint(arm, PICK_ARM);

delay(250);

// 3. Grip Object

Serial.println(F("  -> 3. Gripping object (g180)..."));

moveJoint(grip, GRIP_CLOSE);

delay(400);

// 4. Lift & Rotate Base

Serial.print(F("  -> 4. Rotating base to "));

Serial.print(dropBaseAngle);

Serial.println(F(" deg..."));

moveJoint(arm, arm.homeAngle);

moveJoint(elbow, elbow.homeAngle);

moveJoint(base, dropBaseAngle);

delay(250);

// 5. Drop Object

Serial.println(F("  -> 5. Releasing object (g70)..."));

moveJoint(grip, GRIP_OPEN);

delay(400);

// 6. Return Base Home

Serial.println(F("  -> 6. Returning base Home (b70)..."));

moveJoint(base, base.homeAngle);

delay(200);

Serial.println(F(">>> [COMPLETE] Sort sequence finished successfully.\n"));

return true;

}

void dumpBinLatch(int binIndex) {

if (binIndex == 1) { // Green / Right Bin (CH 12)

Serial.println(F("\n======================================================="));

Serial.println(F("🚪 [AUTO-DUMP] Opening Green Bin Latch (CH 12)..."));

Serial.println(F("======================================================="));

moveJoint(joints[5], LATCH_GREEN_OPEN);

delay(2500);

moveJoint(joints[5], LATCH_GREEN_HOME);

Serial.println(F("🔒 [AUTO-DUMP] Green Latch (CH 12) Closed.\n"));

} else if (binIndex == 2) { // Red / Left Bin (CH 0)

Serial.println(F("\n======================================================="));

Serial.println(F("🚪 [AUTO-DUMP] Opening Red Bin Latch (CH 0)..."));

Serial.println(F("======================================================="));

moveJoint(joints[4], LATCH_RED_OPEN);

delay(2500);

moveJoint(joints[4], LATCH_RED_HOME);

Serial.println(F("🔒 [AUTO-DUMP] Red Latch (CH 0) Closed.\n"));

}

}

void goHomeAll() {

for (int i = 0; i < NUM_JOINTS; i++) {

moveJoint(joints[i], joints[i].homeAngle);

}

}

void setServoAngle(Joint &j, int angle) {

int ticks = map(angle, 0, 180, j.minTicks, j.maxTicks);

pwm.setPWM(j.channel, 0, ticks);

}

void moveJoint(Joint &j, int targetAngle) {

targetAngle = constrain(targetAngle, 0, 180);

if (j.isRelaxed) {

setServoAngle(j, j.currentAngle);

j.isRelaxed = false;

delay(20);

}

if (targetAngle == j.currentAngle) return;

int step = (targetAngle > j.currentAngle) ? 1 : -1;

for (int a = j.currentAngle; a != targetAngle; a += step) {

setServoAngle(j, a);

delay(j.stepDelay);

}

setServoAngle(j, targetAngle);

j.currentAngle = targetAngle;

}